DEPARTMENT OF COMPUTER SCIENCE AND ENGINEERING

23CSE304 Embedded Systems Laboratory

Activity Sheet #6

Activity Name: ADC Module Interfacing

| Name of Student: | Chetan Kumar G |
| --- | --- |
| Registration No.: | CH.SC.U4CSE24109 |
| Activity Date: | 02¦10¦2026 |
| Submission Date: | 02¦10¦2026 |

Assessment Rubrics:

| Component | Maximum Marks | Marks Allotted |
| --- | --- | --- |
| Program | 3 |  |
| Output | 4 |  |
| Viva | 2 |  |
| Timely Submission | 1 |  |
| Total | 10 |  |

Faculty In-charge

Write an embedded C program for ARM architecture to drive ADC module. Debug and verify the program using IDE on a development computer system.

PROGRAM:

#include <LPC21xx.H>

/* LCD control pins */

#define RS  (1 << 0)       // P0.0

#define RW  (1 << 1)       // P0.1

#define EN  (1 << 2)       // P0.2

/* ADC registers */

#define AD0CR  (*((volatile unsigned long *)0xE0034000))

#define AD0GDR (*((volatile unsigned long *)0xE0034004))

void delay(unsigned int t)

{

unsigned int i;

for(i = 0; i < t; i++);

}

/* Send command to LCD */

void lcd_cmd(unsigned char cmd)

{

IO1PIN = (IO1PIN & 0x00FFFFFF) | ((unsigned long)cmd << 24);

IO0CLR = RS;

IO0CLR = RW;

IO0SET = EN;

delay(1000);

IO0CLR = EN;

delay(5000);

}

/* Send data to LCD */

void lcd_data(unsigned char data)

{

IO1PIN = (IO1PIN & 0x00FFFFFF) | ((unsigned long)data << 24);

IO0SET = RS;

IO0CLR = RW;

IO0SET = EN;

delay(1000);

IO0CLR = EN;

delay(5000);

}

/* LCD initialization */

void lcd_init(void)

{

lcd_cmd(0x38);      // 8-bit, 2-line, 5x7

lcd_cmd(0x0C);      // Display ON, cursor OFF

lcd_cmd(0x01);      // Clear display

lcd_cmd(0x06);      // Increment cursor

delay(10000);

}

/* Display string */

void lcd_string(char *str)

{

while(*str)

{

lcd_data(*str);

str++;

}

}

/* Display integer */

void lcd_number(unsigned int num)

{

char buffer[6];

int i = 0;

if(num == 0)

{

lcd_data('0');

return;

}

while(num > 0)

{

buffer[i] = (num % 10) + '0';

num = num / 10;

i++;

}

while(i > 0)

{

i--;

lcd_data(buffer[i]);

}

}

/* ADC read function */

unsigned int ADC_Read(void)

{

unsigned int value;

/*

Select AD0.0

CLKDIV = 4

ADC enabled

*/

AD0CR = (1 << 0) | (4 << 8) | (1 << 21);

/* Start conversion */

AD0CR |= (1 << 24);

/* Wait until conversion completes */

while((AD0GDR & (1UL << 31)) == 0);

/* Extract 10-bit result */

value = (AD0GDR >> 6) & 0x3FF;

/* Stop conversion */

AD0CR &= ~(7 << 24);

return value;

}

int main(void)

{

unsigned int adc_value;

/* P0.27 = AD0.0

Bits 22:23 of PINSEL1 = 01

*/

PINSEL1 &= ~(3UL << 22);

PINSEL1 |=  (1UL << 22);

/* LCD control pins P0.0, P0.1, P0.2 as output */

IO0DIR |= RS | RW | EN;

/* LCD data pins P1.24 - P1.31 as output */

IO1DIR |= 0xFF000000;

/* Initialize LCD */

lcd_init();

/* Display heading */

lcd_cmd(0x80);

lcd_string("ADC VALUE:");

while(1)

{

/* Read ADC */

adc_value = ADC_Read();

/* Move to second line */

lcd_cmd(0xC0);

/* Display ADC value */

lcd_number(adc_value);

/* Clear remaining characters */

lcd_string("     ");

delay(50000);

}

}

OUTPUT WINDOW:

OUTPUT: