DEPARTMENT OF COMPUTER SCIENCE AND ENGINEERING

23CSE304 Embedded Systems Laboratory

Activity Sheet #2

Activity Name: GPIO Switch Interfacing

| Name of Student: | Chetan Kumar G |
| --- | --- |
| Registration No.: | CH.SC.U4CSE24109 |
| Activity Date: | 28¦07¦2026 |
| Submission Date: | 28¦07¦2026 |

Assessment Rubrics:

| Component | Maximum Marks | Marks Allotted |
| --- | --- | --- |
| Program | 3 |  |
| Output | 4 |  |
| Viva | 2 |  |
| Timely Submission | 1 |  |
| Total | 10 |  |

Faculty In-charge

Write an embedded C program for ARM architecture to interface switches. Debug and verify the program using IDE on a development computer system.

PROGRAM:

//CH.SC.U4CSE24109

#include <LPC21xx.h>

#define SW1  (1<<0)      // Switch at P0.0

#define SW2  (1<<1)      // Switch at P0.1

#define LED1 (1<<16)     // LED1 at P0.16

#define LED2 (1<<17)     // LED2 at P0.17

int main(void)

{

// Configure LEDs as output

IODIR0 |= (LED1 | LED2);

// Configure switches as input

IODIR0 &= ~(SW1 | SW2);

while(1)

{

// Switch 1

if(IOPIN0 & SW1)

IOSET0 = LED1;

else

IOCLR0 = LED1;

// Switch 2

if(IOPIN0 & SW2)

IOSET0 = LED2;

else

IOCLR0 = LED2;

}

}

OUTPUT WINDOW:

OUTPUT: