DEPARTMENT OF COMPUTER SCIENCE AND ENGINEERING

23CSE304 Embedded Systems Laboratory

Activity Sheet #4

Activity Name: LCD Interfacing

| Name of Student: | Chetan Kumar G |
| --- | --- |
| Registration No.: | CH.SC.U4CSE24109 |
| Activity Date: | 18¦08¦2026 |
| Submission Date: | 18¦08¦2026 |

Assessment Rubrics:

| Component | Maximum Marks | Marks Allotted |
| --- | --- | --- |
| Program | 3 |  |
| Output | 4 |  |
| Viva | 2 |  |
| Timely Submission | 1 |  |
| Total | 10 |  |

Faculty In-charge

Write an embedded C program for ARM architecture to interface LCD. Debug and verify the program using IDE on a development computer system.

PROGRAM:

#include <LPC21xx.h>

#define LCD_DATA 0x000000FF      // P0.0 to P0.7

#define RS (1<<16)

#define EN (1<<17)

void delay(unsigned int t)

{

unsigned int i,j;

for(i=0;i<t;i++)

for(j=0;j<600;j++);

}

void lcd_cmd(unsigned char cmd)

{

IOCLR0 = LCD_DATA;

IOSET0 = cmd;

IOCLR0 = RS;       // Command mode

IOSET0 = EN;

delay(2);

IOCLR0 = EN;

delay(5);

}

void lcd_data(unsigned char data)

{

IOCLR0 = LCD_DATA;

IOSET0 = data;

IOSET0 = RS;       // Data mode

IOSET0 = EN;

delay(2);

IOCLR0 = EN;

delay(5);

}

void lcd_string(char *str)

{

while(*str)

{

lcd_data(*str++);

}

}

void lcd_init()

{

delay(20);

lcd_cmd(0x38);   // 8-bit, 2-line

lcd_cmd(0x0C);   // Display ON

lcd_cmd(0x06);   // Cursor increment

lcd_cmd(0x01);   // Clear display

}

int main()

{

IODIR0 = LCD_DATA | RS | EN;

lcd_init();

lcd_string("HELLO");

lcd_cmd(0xC0);      // Second line

lcd_string("LPC2138 LCD");

while(1);

}

OUTPUT WINDOW:

OUTPUT: