DEPARTMENT OF COMPUTER SCIENCE AND ENGINEERING

23CSE304 Embedded Systems Laboratory

Activity Sheet #11

Activity Name: UART Programming

| Name of Student: | Chetan Kumar G |
| --- | --- |
| Registration No.: | CH.SC.U4CSE24109 |
| Activity Date: | 02¦10¦2026 |
| Submission Date: | 02¦10¦2026 |

Assessment Rubrics:

| Component | Maximum Marks | Marks Allotted |
| --- | --- | --- |
| Program | 3 |  |
| Output | 4 |  |
| Viva | 2 |  |
| Timely Submission | 1 |  |
| Total | 10 |  |

Faculty In-charge

Write an embedded C program for ARM architecture to drive UART module. Debug and verify the program using IDE on a development computer system.

PROGRAM:

#include <LPC21xx.H>

void delay(unsigned int t)

{

unsigned int i;

for(i=0; i<t; i++);

}

void UART0_Init(void)

{

PINSEL0 |= 0x00000005;   // P0.0 = TXD0, P0.1 = RXD0

U0LCR = 0x83;              // 8 bits, 1 stop, no parity, DLAB=1

U0DLL = 16;                // Baud rate divisor (9600 baud @ 2.5MHz PCLK)

U0DLM = 0;

U0LCR = 0x03;              // DLAB=0, 8-bit mode

}

void UART0_SendChar(unsigned char ch)

{

U0THR = ch;

while(!(U0LSR & 0x20));   // Wait until THR empty

}

void UART0_SendString(char *str)

{

while(*str)

{

UART0_SendChar(*str);

str++;

}

}

int main(void)

{

UART0_Init();

while(1)

{

UART0_SendString("Hello, im chetan\r\n");

delay(1000000);

}

}

OUTPUT WINDOW:

OUTPUT: