DEPARTMENT OF COMPUTER SCIENCE AND ENGINEERING

23CSE304 Embedded Systems Laboratory

Activity Sheet #8

Activity Name: Real-Time Clock Interfacing

| Name of Student: | Chetan Kumar G |
| --- | --- |
| Registration No.: | CH.SC.U4CSE24109 |
| Activity Date: | 02¦10¦2026 |
| Submission Date: | 02¦10¦2026 |

Assessment Rubrics:

| Component | Maximum Marks | Marks Allotted |
| --- | --- | --- |
| Program | 3 |  |
| Output | 4 |  |
| Viva | 2 |  |
| Timely Submission | 1 |  |
| Total | 10 |  |

Faculty In-charge

Write an embedded C program for ARM architecture to interface Real-Time Clock. Debug and verify the program using IDE on a development computer system.

PROGRAM:

#include <LPC21xx.H>

#define RS  (1<<24)

#define EN  (1<<25)

#define LCD_DATA_MASK 0x00FF0000

void delay(unsigned int t)

{

unsigned int i, j;

for(i = 0; i < t; i++)

for(j = 0; j < 1000; j++);

}

void lcd_cmd(unsigned char cmd)

{

IOCLR0 = LCD_DATA_MASK;

IOSET0 = ((unsigned long)cmd << 16);

IOCLR1 = RS;

IOSET1 = EN;

delay(1);

IOCLR1 = EN;

delay(2);

}

void lcd_data(unsigned char data)

{

IOCLR0 = LCD_DATA_MASK;

IOSET0 = ((unsigned long)data << 16);

IOSET1 = RS;

IOSET1 = EN;

delay(1);

IOCLR1 = EN;

delay(2);

}

void lcd_string(char *str)

{

while(*str)

{

lcd_data(*str++);

}

}

void lcd_init(void)

{

IODIR0 |= LCD_DATA_MASK;

IODIR1 |= RS | EN;

delay(20);

lcd_cmd(0x38);     // 8-bit, 2-line

lcd_cmd(0x0C);     // Display ON, cursor OFF

lcd_cmd(0x06);     // Increment cursor

lcd_cmd(0x01);     // Clear display

delay(5);

}

void lcd_num(unsigned int num)

{

lcd_data((num / 10) + '0');

lcd_data((num % 10) + '0');

}

void rtc_init(void)

{

CCR = 0x02;        // Reset RTC

SEC   = 0;

MIN   = 30;

HOUR  = 10;

DOM   = 1;

DOW   = 4;

DOY   = 1;

MONTH = 10;

YEAR  = 2026;

CCR = 0x01;        // Enable RTC

}

int main(void)

{

lcd_init();

rtc_init();

while(1)

{

lcd_cmd(0x80);

lcd_string("TIME: ");

lcd_num(HOUR);

lcd_data(':');

lcd_num(MIN);

lcd_data(':');

lcd_num(SEC);

lcd_cmd(0xC0);

lcd_string("DATE: ");

lcd_num(DOM);

lcd_data('/');

lcd_num(MONTH);

lcd_data('/');

lcd_num(YEAR / 100);

lcd_num(YEAR % 100);

delay(500);

}

}

OUTPUT WINDOW:

OUTPUT: